# CADD STANDARDS MANUAL

FOR

TEXAS TECH UNIVERSITY

LUBBOCK, TEXAS

March 17, 2005

TABLE OF CONTENTS

| 1. 2. 3. 4. 5. 6. 7. 8. 9. 10. 11 | INTRODUCTION CADD SYSTEM DRAWING PLANNING AND SETUP TEXT LAYERS PLOTTING PEN SETTINGS LINETYPES XREF ABBREVIATIONS DRAWING COUTESY APPENDIX A – GRAPHICAL STANDARDS APPENDIX B - LAYER LIST APPENDIX C – ABBREVIATION LIST |  |
| --- | --- | --- |

“The good thing about standards is that there are so many to choose from”

1.	INTRODUCTION

Through the years CADD technology has expanded and grown into a multi-faceted way to store and communicate information.  This CADD manual has been compiled to provide consistent & standard criteria by which all CADD drawings shall be constructed for Texas Tech University Physical Plant Engineering Services (TTUES).

- CADD SYSTEM

TTUES currently operates in the Autodesk 2005 family of products. This family includes, but is not limited to, AutoCAD, ADT, LDT, Map as well as several others. All drawings submitted to TTUES shall be of a compatible format.

- DRAWING PLANNING & SETUP

Drawing Setup

The typical TTUES working drawing shall be setup to take advantage of Paper Space/Model Space. Paper Space shall include the title block and all of the information associated with it. All “sheet specific” information shall be contained in Paper Space as well. Model Space shall contain the objects necessary to convey the design intent. All objects in Model Space shall be drawn at full scale (1:1) and be located at an origin of 0,0,0 unless an accurate survey of the site has been completed. (The lower left hand corner of a building or site shall be at 0,0,0)

A set of templates (*.dwt) have been established to facilitate in the set up of drawings that are to be done by/for TTUES. These templates should be utilized in the creation of all drawings by/for TTUES.

The standard sheet size for projects done by/for TTUES is either 34”x22” or 17”x11”.

Drawing Template

Standard drawing template files have been created according to the standards that are outlined in this manual. These templates are designed to aid in the creation of a standard product. These templates shall be used by everyone internal to TTUES as well as all AE firms working on projects for TTUES.

Template Settings

Titleblock Attributes:

Building

Title of Sheet
Description

Sheet #

# of Sheets

Project Number

Project Manager

Date

Text Style:

Name  TTUES

Font  RomanS.shx

Font Size  3/32"

Dimension Style:

Name  TTUES

Lines and Arrows:

Baseline Spacing  3/32”

Offset  3/32”

Arrowheads:

Size  1/8”

Dimension Lines  Architectural Tick

Leaders  Close Filled Arrow

Text:

Style  TTUES

Placement  Centered

Alignment  Aligned With Dimension Line

Fit:

Scale for dimension features  “Scale dimensions to layout (paper space)”

Primary Units:

Unit Format  Architectural

Precision  1/32”

Special Dimensioning Note

The dimension style in the drawing template is set up to utilize paper space units for scaling purposes.  This allows all of the drawings dimension text to print the same height.  In order to effectively utilize the dimension style set up in the drawing template you must dimension in mspace through the layout tab’s ‘mview’ window.  The result of dimensioning in the model tab is indiscernible text regardless of the scale of the ‘mview’ window.

File Naming Principles

Each file shall contain a whole project (i.e. architectural, mechanical & electrical sheets).  Accordingly files shall be named to fit the following pattern.  First, the discipline code followed by a hyphen, next the sheet sequence identifier followed by an optional user-definable, four-character code.  The following illustration shows the pattern.

A

-

Discipline Code

0

1

Job Number (five digits)

0

1

1

User Definable (customizable)

4. TEXT

All general text on TTUES drawings shall be plotted at a height of 1/16” and be font style RomanS.shx. The only exception to this is the title block, cover and user inserted standard blocks.

5.	LAYERS

The AIA/NIBS National CADD Standard layer names are organized as a hierarchy.  This arrangement allows expansion and addition of user-defined extensions to the layer list.  Layer names are alphanumeric and use abbreviations that are readily identifiable.

Layers shall be named in a fourteen-character system with the first two characters denoting discipline code; the next four characters denoting the major group followed by a hyphen; the following four characters denoting the minor group followed by a hyphen; the final four characters denoting status.

Discipline Code

Major Group

Minor Group

Status (optional)

A

-

W

A

L

L

-

F

U

L

L

-

P

H

S

3

Discipline Code

The discipline code is a two-character field with the second character being a hyphen.  The defined discipline codes are as follows:

A	Architectural

C	Civil

E	Electrical

F	Fire Protection

G	General

H	Hazardous Material

I	Interiors

L	Landscape

M	Mechanical

P	Plumbing

Q	Equipment

R	Resource

S	Structural

T	Telecommunications

X	Other Disciplines

Z        Contractor/shop drawings

Major Group

The major group designation identifies the building system.  Although the major groups are grouped logically with specific codes, it is possible to combine major group codes with any of the discipline codes.  For example, a drawing might contain the following layers:

A-WALL	Architectural Walls

S-WALL	Structural Walls

Minor Group

This is a four-character field for further differentiation of major groups.  For example, A-WALL-PRHT indicates architectural, new, wall, partial height.

Status Field

The status field is a four-character field designator that differentiates new construction from remodeling and existing to remain.  It is only needed when phases of work must be differentiated. A-WALL-PRHT-DEMO indicates architectural, wall, partial height that is to be demolished. Defined values for this field are as follows:

EXST	Existing to remain

DEMO	Existing to be demolished

FUTR	Future work

TEMP	Temporary Work

MOVE	Items to be moved

RELO	Relocated Items

NICN		Not in contract

PHS1-9	Phase numbers

Annotation

Annotation comprises text, dimensions, sheet borders, detail references, and other elements on CADD drawings that don’t represent physical aspects of a building.  Annotation is designated as follows:

*-ANNO-DIMS	Dimensions

*-ANNO-KEYS	Keynotes

*-ANNO-LEGN	Legends and symbols

*-ANNO-NOTE	Notes

*-ANNO-NPLT	Construction lines, non-plotting information

*-ANNO-REDL	Redline

*-ANNO-REVS	Revisions

*-ANNO-SYMB	Symbols

*-ANNO-TEXT	Text

*-ANNO-TTLB	Borders and title blocks

(Note: “*“ represents a discipline code)

User-Definable Fields

The minor group field can be defined by the user to meet their specific needs.  This should only be done if a defined layer does not apply.

Master Layer List

Users may add “user defined” layers in situations where a layer does not exist in this standard to accommodate the need or the layer needed is project specific. These “user defined” layers must follow the conventions set forth by the aforementioned layer guidelines.  This Master Layer List in by no means all-inclusive but shows the most commonly used layer names. The Master Layer List is located in Appendix – B of this manual.

- PLOTTING

- Each layout tab should represent one, and only one, plotted drawing

- A sheet should always be plotted at full scale (1=1) from paper space.

- The origin of the sheet should be located at the lower left, outside corner of the sheet border, not the title block border.

- All layouts need to be plotted using “extents”.

- A sheet should not contain any information placed outside the title block border, with the exception of the plot stamp block

Please note that a guide to pen assignment/line weight and colors can be found in APPENDIX A.

- PEN SETTINGS

A standard CTB file has been established for use with all drawings by/for TTUES. This file is called “TTUES”.

- LINETYPES

Standard linetype formatting shall be adhered to. An example of the standard TTUES linetypes is located in APPENDIX A. The variable “LTSCALE” shall be set to one-half (1/2) of the dimscale. The variable “PSLTSCALE” shall be set to one (1).

- XREF

All drawings by/for TTUES shall utilize xrefs. Xrefs shall include, but is not limited to, titleblocks and floorplans. Xrefs shall be located in the same directory/folder as the drawing files that they are referenced into. This eliminates the need for saving full path. The following diagram illustrates a typical xref scenario:

The variable “VISRETAIN” shall be set to one (1). This allows for layer manipulation in the drawings themselves instead of relying on the xref layer control.

- ABBREVIATIONS

A list of approved abbreviations has been established for use on all drawings by/for TTUES. These approved abbreviations can be located in APPENDIX C of this manual. Periods shall not be used unless the abbreviation can be confused with a whole word. For example if one was to abbreviate Number as No it would require a period to avoid possible confusion.

- DRAWING COURTESY

The following list is an example of things that should be done in every drawing before exiting it and/or submitting it for review.

  - Save the file in the fist layout if multiple layouts are used.

  - Make certain that ONLY the layers that are needed for plotting are left on.

  - Ensure that the layout is zoomed to extents and that there are no extraneous objects outside of the titleblock area.

  - Purge the drawing files to ensure that it is as compact and clean as possible.

  - Set all attribute pulldowns to “by layer”

# APPENDIX A –

# GRAPHICAL STANDARDS

All graphical features of a PPES working drawing shall adhere to the aesthetic examples given in APPENDIX A of this manual.  All graphical representations in APPENDIX A can be found in digital format in the U:\STNDRD directory. The AutoCAD “DesignCenter” may be used in order to insert these features. All annotation objects (i.e. plan identifiers, detail/elevation marks etc.) are scaled according to the standards contained in Appendix A so that they need only be inserted and text edited.

# APPENDIX B –

# DEFINED LAYER NAMES

| LAYER NAME | DESCRIPTION |
| --- | --- |
| Annotation Layers |  |
| *-ANNO-TEXT *-ANNO-REDL *-ANNO-SYMB *-ANNO-LEGN *-ANNO-DIMS *-ANNO-TTLB *-ANNO-NOTE *-ANNO-NPLT *-ANNO-KEYN *-ANNO-REVS | Text Redline Symbols Legends & schedules Dimensions Borders & title blocks Notes Construction lines, non-plotting information key notes revisions |
| Common Modifiers |  |
| *-****-PATT *-****-IDEN *-****-ELEV X-RDME | Cross-hatching, poche Identification tags Elevations Read-me layer, not to be plotted |
| Status Field Modifiers |  |
| *-****-NEWW *-****-EXST *-****-DEMO *-****-FUTR *-****-TEMP *-****-MOVE *-****-RELO *-****-NICN *-****-PHS1-9 | New work Existing to remain Demolition Future work Temporary work Items to be removed Relocated items Not in contract Phase numbers (1-9) |
| Architectural Layers |  |
| A-WALL-FULL A-WALL-PRHT A-WALL-MOVE A-WALL-HEAD A-WALL-JAMB A-WALL-PATT A-WALL-ELEV A-WALL-FIRE A-DOOR A-DOOR-FULL A-DOOR-PRHT A-DOOR-IDEN A-DOOR-ELEV | Full-height walls, stairs and shaft walls, walls to structure Partial-height walls Movable partitions Door & window headers Door & window jambs Wall insulation, hatching & fill Wall surfaces-elevation Fire wall patterning Doors Full-height doors: swing & leaf Partial-height doors: swing and leaf Door number, hardware group, etc. Doors-elevation |

| LAYER NAME | DESCRIPTION |  |  |
| --- | --- | --- | --- |
| A-GLAZ 7 A-GLAZ-FULL A-GLAZ-PRHT A-GLAZ-SILL A-GLAZ-IDEN A-GLAZ-ELEV A-FLOR A-FLOR-OTLN A-FLOR-LEVL A-FLOR-STRS A-FLOR-RISR A-FLOR-HRAL A-FLOR-EVTR A-FLOR-TPTN A-FLOR-SPCL A-FLOR-WDWK A-FLOR-CASE A-FLOR-OVHD A-FLOR-RAIS A-FLOR-IDEN A-FLOR-PATT A-FLOR-PFIX A-FLOR-FIXT A-FLOR-SIGN A-EQPM A-EQPM-FIXD A-EQPM-MOVE A-EQPM-NICN A-EQPM-ACCS A-EQPM-IDEN A-EQPM-ELEV A-EQPM-CLNG A-FURN A-FURN-FREE A-FURN-CHAR A-FURN-FILE A-FURN-PNLS A-FURN-WKSF A-FURN-STOR A-FURN-POWR A-FURN-IDEN A-FURN-PLNT A-FURN-PATT A-FURN-ELEV | Windows, window walls, curtain walls, glazed partitions Full-height glazed walls and partitions Partial-height glazed walls and partitions Windowsills Window number Glazing and mullions-elevation Floor information Floor or building outline Level changes, ramps, pits, depressions Stair treads, escalators, ladders Stair risers Stair & balcony handrails, guard rails Elevator cars and equipment Toilet partitions Architectural specialties (accessories, display cases) Architectural Woodwork (cabinets & counters) Casework (manufactured cabinets) Overhead items (skylights, etc.-usually dashed lines) Raised floors Room numbers, names, targets, etc. Paving, tile, carpet patterns Plumbing fixtures Miscellaneous fixtures Signage Equipment Fixed equipment Movable equipment Equipment not in contract Equipment access Equipment identification numbers Equipment surfaces-elevation Ceiling-mounted or suspended equipment Furniture Free-standing furniture Chairs and other seating File cabinets Furniture system panels Furniture system work surface components Furniture system storage components Furniture system power designations Furniture numbers Plants Finish patterns Furniture-elevations |  |  |
| LAYER NAME | DESCRIPTION |  |  |
| A-CLNG A-CLNG-GRID A-CLNG-OPEN A-CLNG-TEES A-CLNG-SUSP A-CLNG-PATT A-CLNG-ACCS A-LITE A-COLS A-HVAC-SDFF A-HVAC-RDFF A-HVAC-GRID A-ROOF A-ROOF-OTLN A-ROOF-LEVL A-ROOF-STRS A-ROOF-RISR A-ROOF-HRAL A-ROOF-PATT A-ROOF-ELEV A-AREA A-AREA-PATT A-AREA-IDEN A-AREA-OCCP A-ELEV A-ELEV-OTLN A-ELEV-FNSH A-ELEV-CASE A-ELEV-FIXT A-ELEV-PFIX A-ELEV-SIGN A-ELEV-PATT A-ELEV-IDEN A-SECT A-SECT-MCUT A-SECT-MBND A-SECT-PATT A-SECT-IDEN | Ceiling information Ceiling grid Ceiling/roof penetrations Main tees Suspended elements Ceiling patterns Ceiling access Light Fixtures Columns Supply diffusers Return air diffusers HVAC ceiling plan Roof Roof outline Level changes Stair treads, ladders Stair risers Stair handrails, nosing and guardrails Roof surface patterns, hatching Roof surfaces-elevations Area calculations boundary lines Area cross hatching Room numbers, tenant identifications, area calculations Occupant/employee names Interior and exterior elevations Building outlines Finishes, woodwork, trim Wall-mounted casework Miscellaneous fixtures Plumbing fixtures-elevations Signage-elevations Texture & hatch patterns Component identification numbers Sections Material cut by section Material beyond cut section Texture & hatch patterns Component identification numbers |  |  |
| LAYER NAME | DESCRIPTION |  |  |
| A-DETL A-DETL-MCUT A-DETL-MBND A-DETL-PATT A-DETL-IDEN | Details Material cut by section Material beyond cut section Texture & hatch patterns Component identification numbers |  |  |
| Civil Layers |  |  |  |
| C-PROP C-PROP-ESMT C-PROP-BRNG C-PROP-CONS C-TOPO C-TOPO-SPOT C-TOPO-BORE C-TOPO-RTWL C-BLDG C-PKNG C-PKNG-STRP C-PKNG-CARS C-PKNG-ISLD C-PKNG-DRAN C-ROAD C-ROAD-CNTR C-ROAD-CURB C-STRM C-STRM-UNDR C-COMM C-COMM-UNDR C-WATR C-WATR-UNDR C-FIRE C-FIRE-UNDR C-NGAS C-NGAS-UNDR C-SSWR C-SSWR-UNDR | Property lines, survey benchmarks Easements, right-of-way, setback lines Bearings and distance labels Construction controls Proposed contour lines and elevations Spot elevations Test borings Retaining wall Proposed building footprints Parking lots Parking lot striping, handicapped symbols Graphic illustration of cars Parking islands Parking lot drainage slope indications Roadways Centerlines Curbs Storm drainage catch basins, manholes Storm drainage pipe-underground Site communication/telephone poles, boxes, towers Underground communication lines Domestic water-manholes, pumping sta., stor. Tanks Domestic water-underground lines Fire protection-hydrants, connections Fire protection-underground lines Natural gas-manholes, meters, storage tanks Natural gas-underground lines Sanitary sewer-manholes, pumping stations Sanitary sewer-underground lines |  |  |
| LAYER NAME | DESCRIPTION |  |  |
| Electrical Layers |  |  |  |
| E-LITE E-LITE-SPCL E-LITE-EMER E-LITE-EXIT E-LITE-CLNG E-LITE-WALL E-LITE-FLOR E-LITE-OTLN E-LITE-NUMB E-LITE-ROOF E-LITE-SITE E-LITE-SWCH E-LITE-CIRC E-LITE-IDEN E-LITE-JBOX E-POWR E-POWR-WALL E-POWR-CLNG E-POWR-PANL E-POWR-EQPM E-POWR-SWBD E-POWR-CIRC E-POWR-URAC E-POWR-UCPT E-POWR-CABL E-POWR-FEED E-POWR-BUSW E-POWR-NUMB E-POWR-IDEN E-POWR-SITE E-POWR-ROOF E-POWR-OTLN E-POWR-JBOX E-CTRL E-CTRL-DEVC E-CTRL-WIRE E-GRND E-GRND-CIRC E-GRND-REFR E-GRND-EQUI E-GRND-DIAG | Lighting Special lighting Emergency lighting Exit lighting Ceiling-mounted lighting Wall-mounted lighting Floor-mounted lighting Lighting outline for background (optional) Lighting circuit numbers Roof lighting Site lighting Lighting-switches Lighting circuits Luminarie identification and text Junction Box Power Power wall outlets and receptacles Power-ceiling receptacles and devices Power Panel Power equipment Power switchboards Power circuits Underfloor raceways Under-carpet wiring Cable trays Feeders Busways Power circuit numbers Power identification Site power Roof power Power outline for backgrounds Junction box Electric control systems Control systems devices Control system wiring Ground system Ground system circuits Reference ground system Equipotential ground system Ground system diagram |  |  |
| LAYER NAMES | DESCRIPTIONS |  |  |
| E-AUXL E-LTNG E-FIRE E-COMM E-DATA E-SOUN E-TVAN E-CCTV E-NURS E-SERT E-PGNG E-DICT E-BELL E-CLOK E-ALRM E-INTC E-LEGN E-1LIN E-RISR E-SITE E-SITE-LITE E-SITE-UNDR E-SITE-POLE E-SITE-OVHD | Auxiliary systems Lightning protection system Fire alarm, fire extinguishers Telephone, communication outlets Data outlets Sound/PA system TV antenna system Closed-circuit TV Nurse call system Security Paging system Central dictation system Bell system Clock system Miscellaneous alarm system Intercom system Legend of symbols One-line diagrams Riser diagrams Site electrical substations, poles Site lighting Underground electrical lines Electric poles Overhead lines |  |  |
| Fire Protection Layers |  |  |  |
| F-CO2S F-CO2S-PIPE F-CO2S-EQPM F-HALN F-HALN-EQPM F-HALN-PIPE F-IGAS F-IGAS-EQPM F-IGAS-PIPE F-SPRN F-SPRN-CLHD F-SPRN-OTHD F-SPRN-PIPE F-SPRN-STAN F-STAN | CO2 system CO2 sprinkler piping CO2 equipment Halon Halon equipment Halon piping Inert gas Inert gas equipment Inert gas piping Fire protection sprinkler system Sprinkler head-ceiling Sprinkler head-other Sprinkler piping Sprinkler system standpipe Fire protection standpipe system |  |  |
| LAYER NAME | DESCRIPTION |  |  |
| F-PROT F-PROT-EQPM F-PROT-ALRM F-PROT-SMOK | Fire protection systems Fire protection system equipment Fire alarm Smoke detectors/heat sensors |  |  |
| Hazardous Layers |  |  |  |
| H-PLAN H-SITE | Floor plan Site plan |  |  |
| Interior Layers |  |  |  |
| Same as architectural layers with one exception, the discipline code is “I”. For example, I-WALL-FULL is for full-height walls, stairs, shaft walls and walls to structure just as with A-WALL-FULL. |  |  |  |
| Landscape Layers |  |  |  |
| L-PLNT L-PLNT-TREE L-PLNT-GRND L-PLNT-BEDS L-PLNT-TURF L-PLNT-PLAN |  | Plant & landscape materials Trees Ground cover & vines Rock, bark & other landscape bedding Lawn areas Planting plants |  |
| L-IRRG L-IRRG-SPKL L-IRRG-PIPE L-IRRG-EQPM L-IRRG-COVR L-WALK L-WALK-PATT L-SITE L-SITE-FENC L-SITE-WALL L-SITE-STEP L-SITE-DECK L-SITE-BRDG L-SITE-POOL L-SITE-SPRT L-SITE-PLAY L-SITE-FURN |  | Irrigation system Irrigation sprinklers Irrigation piping Irrigation equipment Irrigation coverage Walks & steps Walks & steps-cross-hatch patterns Site improvements Fencing Walls Steps Decks Bridges Pools and spas Sports fields Play structures Site furnishings |  |

| LAYER NAME | DESCRIPTION |
| --- | --- |
| Mechanical Layers |  |
| M-CMPA M-CMPA-CEQP M-CMPA-CPIP M-CMPA-PEQP M-CMPA-PPIP M-CONT M-CONT-THER M-CONT-WIRE M-DUST M-DUST-EQPM M-DUST-DUCT M-ELHT-EOPM M-ENER M-ENER-EQPM M-ENER-WIRE M-RCOV M-RCOV-EQPM M-RCOV-PIPE M-FUME-EXHS M-FUME-EQPM M-EXHS M-EXHS-EQPM M-EXHS-DUCT M-EXHS-RFEQ M-FUEL M-FUEL-GPRP M-FUEL-GGEP M-FUEL-OPRP M-FUEL-OGEP M-HVAC M-HVAC-CDFF M-HVAC-ODFF M-HVAC-DUCT M-HVAC-EQPM M-HVAC-SDFF M-HVAC-RDFF | Compressed air systems Compressed air equipment Compressed air piping Process air equipment Process air piping Controls and instrumentation Thermostats Low voltage wiring Dust & fume collection system Dust & fume collection equipment Dust & fume collection ductwork Electric heat equipment Energy management system Energy management equipment Energy management wiring Energy recovery Energy recovery equipment Energy recovery piping Fume hood exhaust system Fume hoods Exhaust system Exhaust system equipment Exhaust system ductwork Rooftop exhaust system equipment Fuel system piping Fuel gas process piping Fuel gas general piping Fuel oil process piping Fuel oil general piping HVAC system HVAC ceiling diffusers HVAC other diffusers HVAC ductwork HVAC equipment Supply diffusers Return air diffusers |

| LAYER NAME | DESCRIPTION |
| --- | --- |
| M-BRIN M-BRIN-EQPM M-BRIN-PIPE M-CHIM M-HOTW M-HOTW-EQPM M-HOTW-PIPE M-CWTR M-CWTR-PIPE M-CWTR-EQPM M-MACH M-MDGS M-MDGS-EQPM M-MGDS-PIPE M-LGAS M-LGAS-EQPM M-LGAS-PIPE M-NGAS M-NGAS-EQPM M-NGAS-PIPE M-PROC M-PROC-EQPM M-PROC-PIPE M-REFG M-REFG-EQPM M-REFG-PIPE M-SPCL M-SPCL-EQPM M-SPCL-PIPE M-STEM M-STEM-CONP M-STEM-EQPM M-STEM-LPIP M-STEM-HPIP M-STEM-MPIP M-TEST | Brine systems Brine system equipment Brine system piping Prefabricated chimneys Hot water heating system Hot water equipment Hot water piping Chilled water system Chilled water piping Chilled water equipment Machine shop equipment Medical gas systems Medical gas equipment Medical gas pipe Laboratory gas systems Laboratory gas equipment Laboratory gas pipe Natural gas systems Natural gas equipment Natural gas pipe Process systems Process equipment Process piping Refrigeration systems Refrigeration equipment Refrigeration pipe Special systems Special gas equipment Special gas pipe Steam systems Steam condensate piping Steam systems equipment Low pressure steam piping High pressure steam piping Medium pressure steam piping Test equipment |

| LAYER NAME | DESCRIPTION |
| --- | --- |
| Plumbing Layers |  |
| P-ACID P-ACID-PIPE P-DOMW P-DOMW-EQPM P-DOMW-HPIP P-DOMW-CPIP P-DOMW-RISR P-EQPM P-FIXT P-SANR P-SANR-PIPE P-SANR-FIXT P-SANR-FLDR P-SANR-RISR P-SANR-EQPM P-STRM P-STRM-PIPE P-STRM-RISR P-STRM-RFDR | Acid, alkaline, oil waste systems Acid, alkaline, oil waste piping Domestic hot and cold water systems Domestic hot and cold water equipment Domestic hot water piping Domestic cold water piping Domestic hot and cold water risers Plumbing miscellaneous equipment Plumbing fixtures Sanitary drainage Sanitary piping Plumbing fixtures Floor drains Sanitary risers Sanitary equipment Storm drainage system Storm drain piping Storm drain risers Roof drains |
| Equipment Layers |  |
| Q-OTLN Q-POWR Q-PIPE | Equipment outlines Power information Piping information |
| Structural layers |  |
| S-GRID S-GRID-EXTR S-GRID-INTR S-GRID-DIMS S-GRID-IDEN S-FNDN S-FNDN-PILE S-FNDN-RBAR S-SLAB S-SLAB-EDGE S-SLAB-RBAR S-SLAB-JOIN | Column grid Exterior column grid Interior column grid Column grid dimensions Column grid tags Foundation Piles, drilled piers Foundation reinforcing Slab Edge of slab Slab reinforcing Slab control joints |

| LAYER NAME | DESCRIPTION |
| --- | --- |
| S-ABLT S-COLS S-WALL S-METL S-BEAM S-JOIS S-DECK | Anchor bolts Columns Structural bearing or shear walls Miscellaneous metal Beams Joists Structural floor deck |
| Telecommunications Layers |  |
| T-CABL T-EQPM T-JACK T-DIAG | Cable plan Equipment plan Data/Telephone jacks Diagram |

-END OF STANDARD -